Reading the Robot Mind (RTRM)

Reconstructing Inputs from YOLOv5s Layer Activations

Based on: RTRM_for_YOLO_v09d.py

Reference: Nussbaum, P. (2023). Reading the Robot Mind. CLEF 2023.

# 1. Overview and Purpose

The Reading the Robot Mind (RTRM) technique analytically reconstructs input images from intermediate layer activations in a convolutional neural network. Applied here to YOLOv5s, it provides a mathematically grounded window into what each layer of the network "sees" — without training a separate decoder or using gradient-based saliency methods.

The core insight is this: every filter in a convolutional network has an effective receptive field in input-image space. RTRM computes these receptive fields exactly (as RGB patches) and then uses them, weighted by actual network activations, to reconstruct the input.

Nussbaum, P. (2023). Reading the Robot Mind. CLEF 2023 Conference and Labs of the Evaluation Forum.

# 2. YOLOv5s Architecture

YOLOv5s is a single-stage object detector. Its architecture consists of three main sections. All layer indices below refer to model.model[i] in the ultralytics implementation.

| Section | Layer Indices | Components | Role |
| --- | --- | --- | --- |
| Backbone | 0–8 | Focus/Conv, C3 blocks | Feature extraction at multiple scales |
| Neck (FPN/PANet) | 9–19 | Upsampling, Concat, C3 | Multi-scale feature fusion |
| Detection Heads | 20–23 | Detect (3 scales) | Bounding box + class prediction |

## 2.1 Layer 0: Focus vs. Conv

Older YOLOv5 checkpoints use a Focus layer (space-to-depth slicing before convolution). Newer checkpoints use a plain Conv layer. The RTRM code auto-detects which is present.

Source file reference — detection logic:

# Line ~336-360: build_focus_rgb_patches()

conv = self.extract_conv2d(focus_layer)

focus_patches = conv.weight.detach().clone()  # shape: [out_ch, 3, kH, kW]

assert focus_patches.shape[1] == 3  # must be RGB input

Key fact: The Focus conv weight tensor directly encodes the RGB-to-feature mapping. Its shape is [out_channels, 3, kernel_h, kernel_w], where dimension 1 (size 3) confirms it operates on RGB.

## 2.2 C3 Blocks

C3 blocks are the main building unit of the backbone and neck. Each C3 contains three convolutions:

- cv1: main path (feeds into bottleneck sequence)

- cv2: shortcut path (bypasses bottleneck)

- cv3: concatenation point (merges cv1 output + cv2 output)

Source file reference — C3 sub-layer enumeration (lines ~433–480):

if hasattr(layer, 'cv1'):

layers.append({ 'name': f'model.{idx}.cv1', 'is_c3_path': 'cv1', ... })

if hasattr(layer, 'cv2'):

layers.append({ 'name': f'model.{idx}.cv2', 'is_c3_path': 'cv2', ... })

if hasattr(layer, 'cv3'):

layers.append({ 'name': f'model.{idx}.cv3', 'is_concat': True, ... })

## 2.3 Neck: Upsampling and Concatenation

The neck implements a Feature Pyramid Network (FPN) upward path and a Path Aggregation Network (PANet) downward path. Two operation types require special handling in RTRM:

- Upsample layers: 2x nearest-neighbor interpolation (layers 10, 14 in standard YOLOv5s)

- Concat layers: lateral connections from backbone to neck (marker layers only; actual conv is next layer)

Hard-coded concatenation source map (lines ~499–511):

concat_map = {

10: ['model.9',  'model.6'],   # FPN top concat

13: ['model.12', 'model.4'],   # FPN mid concat

16: ['model.14', 'model.10'],  # PANet bot concat

19: ['model.17', 'model.13'],  # PANet top concat

}

Reconstruction note: These indices are specific to the standard YOLOv5s architecture. If you use a custom or pruned model, you must re-derive this map by inspecting model.model[i].f (the 'from' attribute) for each layer.

# 3. Cumulative RGB Patch Building

The RTRM patch-building algorithm computes the effective receptive field (ERF) of each filter in input RGB space. The result is a tensor of shape [out_channels, 3, patch_h, patch_w] for each layer, where each slice [n, :, :, :] is the RGB pattern that maximally activates filter n when centered on the convolution's output.

## 3.1 Mathematical Definition

For a layer with weight tensor W of shape [out_ch, in_ch, fH, fW], and previous layer's patches P of shape [in_ch, 3, pH, pW]:

new_patch[n] = Σᵢ  W[n, i, :, :] ⊗ P[i, :, :]

Where ⊗ denotes spatial placement (not pointwise multiplication): each filter tap W[n,i,fx,fy] multiplies and accumulates P[i] at offset (fx + padding, fy + padding) in the growing patch canvas.

New patch spatial dimensions:

new_h = prev_h + 2 * padding + (fH - 1)

new_w = prev_w + 2 * padding + (fW - 1)

Source: build_rgb_patch_standard(), lines ~546–620

## 3.2 GPU-Vectorized Implementation

The inner loop is vectorized using torch.einsum over filter spatial positions, avoiding the O(out_ch × in_ch) double loop on CPU:

# Lines ~597–614 (GPU path)

for fx in range(fh):

for fy in range(fw):

weights_at_pos = filters[:, :num_to_use, fx, fy]  # [out_ch, in_ch]

weighted_contribution = torch.einsum(

'fi,icxy->fcxy',

weights_at_pos,         # [num_filters, num_input_ch]

prev_patches[:num_to_use]  # [num_input_ch, 3, pH, pW]

)

new_patches[:, :, x_start:x_end, y_start:y_end] += weighted_contribution

The einsum string 'fi,icxy->fcxy' contracts over i (input channels), producing [f=out_filters, c=3 RGB, x, y]. This is the critical line for reproducibility.

## 3.3 Focus Layer: First-Layer Patches

The Focus/Conv layer is the only one that takes raw RGB as input, so its patches are simply the conv weights themselves — no cumulative accumulation is needed. This is the boundary condition for the recursion.

# Lines ~336–360: build_focus_rgb_patches()

focus_patches = conv.weight.detach().clone()

# Shape: [out_channels, 3, kH, kW] — already in RGB patch format

## 3.4 Upsampling Patches

When the network upsamples feature maps (2x nearest neighbor in the neck), the patches must be upsampled identically to remain spatially consistent:

# Lines ~379–399: upsample_patches()

upsampled = torch.nn.functional.interpolate(

patches, scale_factor=factor, mode='nearest'

)

## 3.5 Concatenation: Align, Concat, Build

When patches from two sources are concatenated (neck lateral connections), the procedure is: (1) align both sets to the same spatial size via center-crop or zero-pad, (2) concatenate along the channel dimension (dim 0), (3) proceed with standard patch building using the merged tensor as prev_patches.

# Lines ~637–675: concatenate_source_patches()

# Step 1: align spatial dims to max(h), max(w)

max_h = max(p.shape[2] for p in source_patches_list)

max_w = max(p.shape[3] for p in source_patches_list)

# Step 2: concatenate along dim 0 (channel dimension)

concatenated = torch.cat(aligned_sources, dim=0)

# Step 3: standard patch build with concatenated as prev_patches

Version history note: Versions 8a/8b had overcomplicated split-and-add logic for concatenation. Version 8c simplified this to the align→concat→standard-build pattern above. If you find older code with 'split_and_add', it is incorrect.

# 4. Image Reconstruction from Activations

Given the cumulative RGB patches for a layer and the layer's activation map from a real image, reconstruction proceeds by transposed convolution: each activation value weights its corresponding patch, and all weighted patches are summed into a single RGB canvas.

## 4.1 Core Reconstruction Formula

reconstruction = Σₙ  activation[n] × patch[n]

Implemented as a single transposed convolution:

# Lines ~828–839: reconstruct_rgb_standard()

reconstruction = torch.nn.functional.conv_transpose2d(

input=activations,   # [batch, num_filters, act_h, act_w]

weight=patches,      # [num_filters, 3, patch_h, patch_w]

bias=None,

stride=stride,       # matches the original conv stride

padding=0

)

## 4.2 Stride-Aware Reconstruction

For layers with stride 2 (downsampling), the transposed convolution must use stride=2 to correctly expand the activation grid back to the spatial scale of the reconstruction. This is the mathematical inverse of strided convolution.

The stride value for each layer is extracted from the Conv2d object:

stride = conv.stride

if isinstance(stride, tuple): stride = stride[0]  # assume square

## 4.3 Adaptive Cropping

Each stride-2 layer shrinks the spatial extent of the feature map. After reconstruction, border artifacts accumulate proportionally. The code removes a border of cumulative_stride2_count pixels on each side:

# Lines ~843–852

if cumulative_stride2_count > 0:

crop_pixels = cumulative_stride2_count

reconstruction = reconstruction[

:, :,

crop_pixels:-crop_pixels,

crop_pixels:-crop_pixels

]

cumulative_stride2_count is computed by scanning layer_sequence and incrementing a counter each time stride==2 is encountered (lines ~944–948).

## 4.4 COCO Normalization

After spatial reconstruction, the output is normalized to match the COCO/ImageNet channel statistics. This substantially improves visual contrast versus naive min-max scaling.

# Lines ~791–806: normalize_reconstruction_coco()

target_mean = [0.485, 0.456, 0.406]  # COCO per-channel means (RGB)

target_std  = [0.229, 0.224, 0.225]  # COCO per-channel stds  (RGB)

# Standardize then re-scale:

recon = (recon - current_mean) / current_std

recon = recon * target_std + target_mean

recon = torch.clamp(recon, -0.5, 1.5)  # clip only extreme outliers

## 4.5 C3 Block Reconstruction: Merging Two Paths

Because cv1 and cv2 both originate from the same input image, their reconstruction contributions must be summed (not kept separate). The procedure:

- Reconstruct cv1 path WITHOUT normalization (use_coco_norm=False)

- Reconstruct cv2 path WITHOUT normalization

- Add the two unnormalized reconstructions together

- Apply COCO normalization once to the merged result

# Lines ~874–915: reconstruct_cv3_merged()

merged = recon_cv1 + recon_cv2

merged = self.normalize_reconstruction_coco(merged)

Unnormalized intermediate reconstructions are stored in reconstruction_cache with keys like 'model.2.cv1_unnorm' so they can be reused if cv3 for the same C3 block is processed again.

# 5. Detection-Specific Reconstruction (v09a/v09d)

Beyond layer-wise reconstruction of the full image, RTRM can isolate what the network processed for each individual detected object. Two methods are implemented:

## 5.1 Method 1: Spatial Masking

For each bounding box, the activation map is zeroed everywhere except a 3x3 spatial region centered on the detection's grid cell. Reconstruction from this masked activation shows only the input features that contributed to that detection's spatial position.

# Lines ~1015–1040: reconstruct_from_detections()

# Map bbox center to activation coordinates:

act_x = int((bbox_center_x / 640) * act_w)

act_y = int((bbox_center_y / 640) * act_h)

# Create masked activation (zero everywhere except 3x3 window):

masked_activations = torch.zeros_like(full_activations)

masked_activations[:, :, y_start:y_end, x_start:x_end] = \

full_activations[:, :, y_start:y_end, x_start:x_end]

## 5.2 Method 2: Class-Specific Filter Selection (v09d)

Version 9d introduced a second method that restricts reconstruction to the filter channels associated with the detected class. The 80 COCO classes are mapped to non-overlapping subsets of the filter bank:

# Lines ~1115–1135: reconstruct_from_detection_heads()

filters_per_class = num_filters // 80  # partition filters among 80 COCO classes

class_start = cls_id * filters_per_class

class_end   = min((cls_id + 1) * filters_per_class, num_filters)

# Zero all filters outside [class_start, class_end]:

masked_activations = torch.zeros_like(full_activations)

masked_activations[:, class_start:class_end, y_start:y_end, x_start:x_end] = \

full_activations[:, class_start:class_end, y_start:y_end, x_start:x_end]

Interpretation caveat: This filter-to-class assignment is a heuristic — the network does not literally partition its filters by class. The intent is to highlight that different filter subsets produce visually different reconstructions, illustrating that spatial position alone does not fully explain a detection.

# 6. Capturing Layer Activations via Forward Hooks

Activations are captured at runtime using PyTorch forward hooks, without modifying the model. A hook is registered, a forward pass is run, and the hook is immediately removed.

# Lines ~754–788: get_layer_output()

def hook_fn(module, input, output):

self.activation_cache[layer_name] = output.detach()

handle = layer.register_forward_hook(hook_fn)

with torch.no_grad():

_ = self.model(input_tensor)

handle.remove()

Layer name parsing rules (lines ~757–776):

| Layer name format | Example | Parsing logic |
| --- | --- | --- |
| 'model.0.Focus' | Focus layer | self.model.model[0] |
| 'model.N' | 'model.9' | self.model.model[int(N)] |
| 'model.N.cvX' | 'model.2.cv1' | getattr(self.model.model[int(N)], 'cvX') |

# 7. Image Preparation and Letterboxing

YOLOv5 expects 640x640 square input. The code uses letterboxing: scale the image to fit within 640x640 while preserving aspect ratio, then pad the remaining area with pixel value 114 (a neutral gray).

# Lines ~198–228: load_and_prepare_image()

scale = min(640 / h, 640 / w)          # scale to fit

new_h, new_w = int(h*scale), int(w*scale)

prepared_image = np.full((640, 640, 3), 114, dtype=np.uint8)  # gray canvas

pad_h = (640 - new_h) // 2

pad_w = (640 - new_w) // 2

prepared_image[pad_h:pad_h+new_h, pad_w:pad_w+new_w] = resized

The padding value 114 is the standard YOLOv5 letterbox fill. Using a different value will shift pixel statistics and may slightly affect normalization quality in reconstructions.

# 8. Output Files

| File | Content | DPI |
| --- | --- | --- |
| rtrm_v09d_rgb_filter_patches_all_layers.png | Grid of first 6 RGB patches for every layer (backbone + neck) | 150 |
| rtrm_v09d_reconstruction_<imagename>.png | 5-column grid of per-layer reconstructions, depth-ordered | 300 |
| rtrm_v09d_dual_detection_<imagename>.png | 2x2 comparison: spatial masking vs class-filter methods, with/without bounding boxes | 200 |
| prepared_<imagename>.jpg | 640x640 letterboxed input used for all forward passes | — |
| detection_results.jpg | Annotated detection image from test run | — |

# 9. Dependencies and Setup

pip install torch ultralytics numpy matplotlib opencv-python pillow

The ultralytics package provides the YOLO wrapper. The underlying model is accessed as:

model_wrapper = YOLO('yolov5s.pt')

model = model_wrapper.model   # nn.Module with .model[i] layer access

Model is automatically downloaded from Ultralytics on first run. Place your own images in a './data' directory; otherwise the script downloads sample images from ultralytics.com.

# 10. Reproducibility Checklist

- Random seeds: torch.manual_seed(42) and np.random.seed(42) set at module level (lines 76–77)

- Device: model and tensors moved to CUDA if available, else CPU

- Determinism: all forward passes run under torch.no_grad()

- Patch building: no randomness; pure linear algebra on pretrained weights

- COCO normalization constants: mean=[0.485,0.456,0.406], std=[0.229,0.224,0.225]

- Clamp range after normalization: [-0.5, 1.5] (deliberately wider than [0,1] to preserve contrast)

Source: RTRM_for_YOLO_v09d.py  •  Reference: Nussbaum, P. (2023). Reading the Robot Mind. CLEF 2023.